Training Circular No. 6 of 2005

| No:11-TR(49)/2004 | Dated: January 1, 2005 |
| --- | --- |
| Subject: Guidelines - Ship Manoeuvring Simulator & Bridge Team work course |  |
| Maritime safety and efficiency is of utmost concern to the Directorate General of Shipping (DGS), the constituted authority of the Government of India for all maritime affairs. It was noticed that some training institutes barely met the minimum standards set by the Directorate, while others provided training facilities that were world class. In order to bring about uniformity of training, amenities and equipment provided by all training institutes the guidelines were revised and new ones formulated. This was done by the DGS in consultation with the training institutes and employers (shipowners and manning agencies). Draft guidelines were displayed on the website of the DGS and comments invited from interested parties, regardless of the position or office held. Comments received were considered to the earliest possible. The final guidelines for Course is attached herewith. In keeping with Government policy, the guidelines have been made in as transparent manner as practicable. It is expected that training institute would follow these guidelines in letter and spirit. The above guidelines shall come into force w.e.f. 01.01.2005. This issues with the approval of the Director General of Shipping and ex-officio Addl. Secretary to the Government of India. Sd/- (Naresh Salecha) Sr.Dy. Director General of Shipping |  |
| MANDATORY GUIDELINES FOR TRAINING INSTITUTES FOR OBTAINING APPROVAL FROM DIRECTORATE GENERAL OF SHIPPING TO CONDUCT # SHIP MANOEUVRING SIMULATAR & BRIDGE TEAM WORK To avoid unnecessary repetition, reference has been made herein to DGS Order no: 1 of 2003 (Guidelines for the conduct of Pre-Sea Training courses for Merchant Navy) wherever appropriate. TABLE OF CONTENTS 1. BASIC DETAILS OF THE COURSE 1.1. Aims: 1.2. Objectives: 1.3. Application:. 2. QUALIFICATION & ELIGIBILITY OF STUDENTS 2.1. Entry standards: 2.2. Required attendance: 2.3. Course intake limitations:. 3. INFRASTRUCTURE REQUIREMENT. 4. COURSE DETAILS 4.1. Course duration 4.2. Course outline 4.3. Detailed Teaching Syllabus. 5. HOLIDAYS. 6. FACULTY REQUIREMENT 6.1. Qualifications and experience of course in charge: 6.2. Qualifications and experience of faculty members: 6.3. Training of Trainers & Assessors Course: 6.4. Visiting faculty members: 6.5. Age limit for regular faculty members:. 7. FACULTY STRENGTH. 8. COURSE DURATION. 9. ASSESSMENT. 10. QUALITY STANDARDS. 11. INSPECTIONS. 12. COST OF INSPECTIONS. 13. FEES TO GOVT.. TEACHING AID. Annexure - 1 - Additional equipment, etc. Annexure - 2 – Course outline. Annexure - 3 - Course Time table.. Annexure - 4 - Course Certificate. SHIP MANOEUVRING SIMULATOR AND BRIDGE TEAM WORK 1. BASIC DETAILS OF THE COURSE 1.1. Aims: This course covers the training recommended in the IMO Model Course 1.22 and fulfills the training requirement for Navigating officers at the Management Level, specified in Table A-II/2 of STCW Code 1995 and Appendix M-II/3B of META Manual Vol 2, concerning manoeuvring and handling a ship in various conditions and establishing good team work during Bridge Watchkeeping. 1.2. Objectives: The trainee who successfully completes this course will have gained experienced in handling ships under various conditions and will make a more effective contribution to the bridge team during ship manoeuvring in normal and emergency situations. in particular, trainees will gain: 1.2.1 Familiarization with the use of engines and helm for ship manoeuvring. 1.2.2 An understanding of the effects on the behaviour of the ship of wind, current shallow water, banks and narrow channels and condition of loading. 1.2.3 A greater awareness of the importance of planning in passage or manoeuvre and the need for an alternative plan, 1.2.4 A greater understanding and awareness of efficient bridge procedures and bridge teamwork during watchkeeping and ship handling, in normal and in emergency situations, 1.2.5 A greater awareness and understanding of a good interactive communication style and benefit of building up a common shared mental model of the planned passage. 1.3. Application: These guidelines shall be applicable from. 1st Jan, 2005. 2. QUALIFICATION & ELIGIBILITY OF STUDENTS 2.1. Entry standards: The trainee should be in possession of  Chief Mate (F.G.) Certificate of Competency or Mate (H.T.) Certificate of Competency and sea-service of at least  18 months as a certified watch keeping officer. 2.2. Required attendance: 100% attendance is required for successful completion of the course. However, in exceptional circumstances, a student is allowed absence of up to one day subject to his attending the lectures missed out during the next course at the same institute. The institute shall keep proper records of such cases. 2.3. Course intake limitations: The course intake shall be a maximum of 8 trainees divided into two bridge teams. Depending on the facilities provided, one team would be carrying out an exercise while the other is being lectured, debriefed or planning the next exercise. 3. INFRASTRUCTURE REQUIREMENT 3.1 Full Mission Ship Handling Simulator The Ship-Handling simulator shall be type approved by DG shipping on demonstrating its capability of fulfilling the required “Specification for Ship Manoeuvring Simulator” (Annex 1) ###### 3.2 Classroom A briefing room capable of seating 4 trainees + instructor as per 1.2 sq.m. per trainee size requirement, with a briefing table. The briefing room shall be equipped with a white / black board, overhead / LCD projector and a capability of projecting recording of ship tracks and actions taken during the exercise for the purpose of debriefing. 4. COURSE DETAILS 4.1. Course duration: 5 days 4.2. Course outline: As per Annexure 2 5. HOLIDAYS 5.1. Sundays shall be holidays. 5.2. Independence Day (15th August) and Republic Day (26th January) shall be compulsory holidays. 5.3. Students shall normally enjoy the holidays observed by the Govt of the state in which the institute is located. 6. FACULTY REQUIREMENT 6.1. Qualifications and experience of course in charge: 6.1.1. Certificate of competency as Master of (FG), issued or recognised by the Government of India. and 6.1.2. At least 5 years service on Merchant ships of which at least two years should have been in the rank of Master And Should have undergone training of similar or equivalent course on a full mission ship handling simulator. 6.2 Qualifications and experience of faculty members: 6.2.1 Certificate of competency as Master of (FG), issued or recognised by the Government of India. and 6.2.2 At least 5 years service on Merchant ships of which at least one year should have been at the rank of Master. and 6.2.3 Should have undergo training of similar or equivalent course on a full mission ship handling simulator. 6.3. Training of Trainers & Assessors Course: As per DGS Order no: 1 of 2003. 6.4. Visiting faculty members: Qualifications and experience of visiting faculty members should be the same as that of regular faculty as specified above. 6.5. Age limit for regular faculty members: As per DGS Order no: 1 of 2003. 7. FACULTY STRENGTH 7.1 Not less than one Master Mariners (inclusive the course-in-charge) in case where number of participants are 4 or less and not less than two Master Mariners (inclusive of the course-in-charge) in case the number of participants are 5 or more. 7.2 A minimum of 50% of the entire portion must be covered by permanent faculty. 8 COURSE DURATION A total of 35 hours of lectures, including practical training and assessment. 9 ASSESSMENT Assessment would be carried out at the end of each course. 10 QUALITY STANDARDS As per DGS Order no: 1 of 2003. 11 INSPECTIONS As per DGS Order no: 1 of 2003. 12 COST OF INSPECTIONS As per DGS Order no: 1 of 2003. 13 FEES TO GOVT. As per DGS Order no: 1 of 2003. 14. TEACHING AIDS 14.1. Two sets of charts for the exercise areas used in the course. 14.2. 1 set of Tide tables, List of Lights, Sailing Directions for the exercise areas in use. These should relate to the simulator software and need not be the latest edition. 14.3. Ship Log Book – 2 sets 14.4. Passage Planning Sheets 14.5. Booklet containing Ship Maneuvering Data of the ships in use – 2 sets 14.6. Equipment Manuals for the Navigational Aids fitted on the bridge 15. Recommended Textbooks 15.1 The Ship Handler’s Guide by Capt. R. W. Rowe (N.I.) 15.2 Bridge Procedures Guide by International Chamber of Shipping 15.3 International Regulations for Preventing Collisions at Sea (COLREG 1972) as amended by IMO 15.4 Perils at Sea and Salvage – A guide for Masters by ICS & OCIMF 15.5 Bridge Team Management by Capt. A. J. Swift (N. I.) 16. Reference Book 16.1 Collisions & their causes – Capt. Richard A. Cahill. (N.I) 16.2 Maritime Resource Management - Student workbook developed by SAS Flight Academy, Sweden. 16.3 A Guide to Collision Avoidance Rules by Cockcroft and Lameijer 16.4 Ship Manoeuvring principles and Pilotage by Paul R. Williamson 16.5 The Navigation Control Manual by A. G. Bole, W. O. Dineley & C. E. Nicholls. 17. IMO References 17.1 Performance Standards for Automatic Radar Plotting Aids (ARPA) – IMO Resolution A.823(19) 17.2 Performance Standards for Gyro-Compass – IMO Resolution A.424 (XI) 17.3 Performance Standards for Devices to indicate Speed and Distance – IMO Resolution A.824(19), MSC 96(72) 17.4 Performance Standards for Integrated Bridge Systems – MSC Resolution 64 (67) 17.5 Performance Standards for DGPS – MSC Resolution 64 (67), 114(73) 17.6 Performance Standards for Automatic Pilots – Resolution A.342 (IX) 17.7 Performance Standards for Radar Equipment – Resolution A.477 (XII) 17.8 Performance Standards for ECDIS - Resolution A.817 (19) 18. Recommended Videos 18.1 Ship Handling – Parts 1, 2, 3 - Videotel and NMI 18.2 Ship Handling in restricted waters - Videotel - ship squat and shallow water effects 18.3 Passage Planning - Videotel 18.4 Bridge Watchkeeping - Videotel 18.5 Master / Pilot Relationship - Videotel 18.6 Accident Prevention and Human Factor - Videotel 18.7 Emergency Procedures - Videotel 18.8 Bridge Team Management - The Standard P & I 18.9 No Room for Error - UK P & I 19. Time-table Timetable is made as per two bridge Teams labelled Team A and Team B See Annex 3 Annexure 1 SPECIFICATIONS FOR SHIP MANOEUVRING SIMULATOR The Simulator should conform to the requirements of STCW 95 Regulation I/12 (use of simulators), Section A — I/12 Part 1 and 2 “Perform Standards for the simulator and Simulator training Objectives” and section B — I/12, 37, 38, 39 (guidance regarding the use of simulators). It should also be capable of being used for training and demonstrating competence for Masters and Chief Mates on ship of 500 GRT or more as per STCW 95 A-II/2 SIMULATOR SPECIFICATIONS - Full - Mission Simulator consisting of one or more own ship stations having a full scale mock - up of a ship’s bridge with instruments for navigation as listed below, as well as full scale display of target ships and surroundings as seen from the portholes of a wheel house. - Equipment and consoles to be installed, mounted and arranged in a ship like manner. - A separate instructor room equipped with equipment necessary to monitor the activities in the wheel house effectively. - Each piece of equipment installed in the simulator shall have a similar functionality to corresponding equipment used on board ships. - If any piece of equipment does not correspond to a specific make, (the applicable IMO performance standard (functionality requirements only) for such equipment shall be followed. If such a performance standard does not exist, then the functionality of the equipment shall, as a minimum, be the same as for any recognized genuine equipment of that type, in use on board ships. - Each piece of equipment shall resemble the behavioral characteristics e.g. accuracy, reaction time and other limitations, related to corresponding equipment in use on board ships. - User manuals for the simulator equipment and operational controls shall be available to the learners for use during exercises. SHIP TYPES AND AREAS 1. The simulator shall include mathematical models of at least 10 types of own ship. The models shall resemble accurately the behavioral characteristics of an actual ship of that size, power and type, and realistically behave as per the hydrodynamic effects of wind, current and swell. (See section E for recommended ship types) 2. The simulator shall be able to present at least 20 different types of target ships, each equipped with a mathematical model, which accounts for motion, drift and steering angles according to forces induced by current, wind and wave. 3. The simulator should be able to provide at least eight international geographical visual areas for exercises which include open sea and high density traffic areas. (See section F for recommended Geographical Areas) DETAILED SPECIFICATIONS A. Visualization 1. At least 5 channels visualization of high resolution XGA graphics, about 170 degree horizontal field of view. In addition, the remaining 190 degree view should be able to be viewed by panning. 2. The visual screen size for each channel shall not be less than 6’ x 4’. The visual system shall present all navigational marks as displayed on ECDIS and paper charts for that area. 3. The visual system shall show objects with sufficient realism (detailed enough to be recognized as in real life). 4. The visual system shall replicate movements of all Own Ships according to 6 degrees of motion freedom. 5. The simulator shall provide a realistic visual scenario by day, dusk or by night, including variable meteorological visibility, changing in time. It shall be possible to create a range of visual conditions, from clear to dense fog. 6. It shall be possible to take accurate bearings of objects seen on the screen. 7. It shall be possible to use magnified view for observations. 8. The visual system shall present at least 25 degrees of vertical field view. In addition by any method, it shall be possible to observe the ship’s side and the dock during mooring operations. 9. There should be a proper correspondence between the visual picture, radar and ECDIS. B. Simulator Capabilities 1. The model shall realistically simulate own ship hydrodynamics in open water conditions, including the effects of wind forces, wave forces, tidal stream and currents. 2. The model shall realistically simulate own ship hydrodynamics in restricted waterways, including shallow water and bank effects and interaction with other ships. 3. The simulator shall provide an own ship engine sound reflecting the power output. 4. The target ships shall be equipped with navigational — lights, shapes and sound signals, according to the “Rules of the Road”. The signals shall be individually controlled by the instructor and the sound signals shall be directional and fade with range. 5. The simulator shall be able to present at least 20 target ships (See STCW-95 Section A-I/12.4.3) at the same time, where the instructor shall be able to program voyage routes for each target ship individually. 6. The simulator shall be capable of providing environmental sound (e.g. wind) according to conditions simulated. 7. The simulation shall include the depth according to charts used, reflecting water level according to tidal water situation. 8. The simulator shall provide waves, variable in direction and strength. 9. It shall be possible to simulate usage of at least 4 tugs for the purpose of mooring the vessel with the capability to control the power and orientation of the tugs (push and pull). 10. It shall be possible to berth and unberth a vessel using mooring lines with the capability to control run out, heave, slack, stop, let go the various mooring lines bearing in mind their breaking stress. 11. It shall be possible for Own ship to let go the bower anchors and control is pay-out as per the strain on the cable. The simulator shall have the capability to read the number of shackles out and the strain at any time. C Own Ship Control Station The following shall be provided as HARDWARE PANELS and shall be installed, mounted and arranged in a manner that would physically resemble a ship’s navigating bridge. These hardware panels should have operational resemblance to actual shipboard equipment. 1. Propulsion Controls for controlling own ships engine ahead and astern. 2. Bow-Thruster Control 3. Steering Console Stand There shall be provision for the following, at or near the console: (a) Steering wheel (b) Steering motors (at least two) (c) Hand, auto-pilot and non-follow up steering. (d) Compass Repeater able to depict gyro and/or magnetic heading. (e) Gyro failure alarm (f) Auto-pilot The Auto-Pilot should have the following capabilities :- i) Weather adjustment (yawing and course control) ii) Rudder limit setting iii) Counter Rudder iv) Off-course alarm v) Setting of constant rate of turn. 4. Engine Alarm Panel giving audible and visual alarm in case of :- (a) Start fail (b) Shutdown (C) Slow down (d) Overspeed (c) Overload 5. Radar set with Automatic Radar Plotting Aids (ARPA) - 21” colour screen It shall be possible to simulate both 3 cms and 10 cms radar. The radar shall be capable of being operated in the sea stabilized relative motion mode and sea and ground stabilized true motion modes (see STCW-95 Section A-I/12.4.1 and 3). The radar simulation equipment shall be capable of generation of interference, noise, Radar/ARPA failure, yawing, clutter, spurious echoes, blind sector, parallel index lines. The ARPA simulation equipment shall incorporate the facilities for:- - Manual and automatic target acquisition. - Past track information - Use of exclusion areas - Vector/graphic time and data display - Trial manoeuvres 6. ECDIS — 21” colour screen Vector charts should be available for the exercise areas. It should be possible to edit existing areas and be able to generate chart database of any area and scale if desired at a later stage. Normal features for ECDIS system should be available including cart scaling and zooming, review, selectable layer, route planing and monitoring. 7. VHF Communication System Communication between ships and port VTS shall be simulated on VHF sets which will have at least the following channels = 16, 6, 8, 9, 10, 12, 13, 14, 75, 77, 69, 67. The following realism should be depicted - Volume control - Squelch - Dual watch. - Pressel switch when speaking - Simplex communication system. 8. lntercom / Telephone There should be a provision to communicate between Bridge and the other strategic locations like engine room, steering flat, Master, C/O, 2/O, 3/O, C/E, forward, aft, etc. 9. General Emergency Alarm There shall be a facility provided for activating the General Emergency Alarm from the wheelhouse. 10. Chart Table with paper charts and publications Chart Table will resemble a ship’s Chart Table of minimum dimension 4’ x 3’. Paper charts and publications provided shall be appropriate for the areas in use. 11. Indicators Each own ship station shall have atleast the following indicators:- (a) Wind direction and speed indicator (b) Rudder Angle Indicator (c) Rate of Turn Indicator (d) RPM /Pitch Indicator (e) Clock (Exercise time indicator) (f) Depth indicator (g) Doppler Speed Log It should be capable of indicating fore / aft and athwartship speed. Depending upon the depth, speed shall be indicated on ground or water track. 12. Ships Horn To be provided on the wheelhouse console as a pushbutton. The following equipment shall be SIMULATED:- If not using hardware panels, then, a colour monitor of not less than 17” size interfaced with the position and movement of own ship shall be used. 1. Electronic Navigation Aids (a) Global Position System (GPS) - Simulation of all facilities of a standard GPS receiver shall be available. This should include display of latitude, longitude, course and speed over ground by the own ship, UTC, normal navigational calculation functions such as Great Circle, Rhumb line sailing, 100 way points, Alarms for X-track error, anchor drag, approaching way point, etc. 2 Echo Sounder Simulation of complete echo sounder shall be provided. Facility to change gain adjustment, change over from DBS to DBK and vice versa etc, shall be provided. Alarm for shallow water depth should be provided. 3 Anchor Control capable of simulating anchoring with 2 anchors (port and stbd - Bower anchors) (a) Means to let go and heave up own ship’s anchor (b) Indicators for amount of cable paid out, direction of cable and strain on cable. 4. Sound Signal Generator Ship’s whistle and fog signals: Facilities shall be available to generate fog signals manually or automatically operated by own ship(s) independently, as well as for each target separately by the Instructor console. The fog signals should be interactive and the intensity and direction at own ship stations shall correspond to relative range and position of the station generating the sound signal. The fog signal generator shall be capable of generating the sound signals for the following:- a) Vessel making way through water. b) Vessel making no way through water. c) Vessel restricted in her ability to maneuver d) Vessel at anchor e) Vessel aground f) Vessel not under command. 5 Navigation Lights and Shapes Display Full set of Navigation, Christmas tree lights and shapes shall be available, which the own ship can select for display depending upon the prevailing circumstances. D Instructor The instructor and the assessor shall be able to: a, Start, halt, reset in time and place, and restart an exercise b. Visually observe the trainees and their actions and follow the proceedings of an exercise by any method. c. Change the operating environment during the running of an exercise, viz. shall he able to alter the wind (direction and force), swell (direction and height), current (direction and rate), cloud cover, state of visibility. d. Communicate with the trainees (i e. simulate the outside world) by VHF on relevant communication channels and by Intercom / Telephone (for within the ship conversations) e. A display (min 19” monitor) providing a global view of the criteria simulation scenario. The display plots ships tracks, target movements and also provides a tool for altering the parameters of the various ships. f. Be able to view the Ownship radar as set and operated by the trainee. g. Activate simulation of failures in real time in the following equipment :- i) Navigation lights ii) Gyro compass including insertion of error iii) Doppler log failure or insertion of error iv) Echo sounder v) Radar vi) ARPA vii) GPS (including degrading of signal quality) viii) Autopilot ix) Steering motor x) Bow thruster xi) Engine h. It shall be possible to replay a full exercise showing the actions performed by the trainees. i. Instructor shall be able to create exercises where one or more own ship stations can be interactive within the exercise or to be able to run them independently and in differing areas if so required. j. For educational purposes, the instructor shall be able to create a channel by inputing depths and buoys (buoyage system A and / or B) k. Instructor can on request from Own ship, engage tugs and ship mooring lines during an exercise. E Recommended Ship Types and Sizes Type; Displacement (tonnes). 1.; Bulk Carrier (Handy size); About 30,000. 2.; Bulk Carrier (Panamax size); About 60,000. 3.; Container Ship; About 30,000. 4.; Container Ship; About 70,000. 5.; Container Ship; About 1,00,000. 6.; Coaster; Under 10,000. 7.; RO-RO / Car Carrier; About 15,000. 8.; Tanker; About 85,000. 9.; Super tanker; Over 1,50,000. 10.; VLCC; Over 2,50,000. Note: A. The above is a suggested list for general guidance B. Among the Own Ship Types, there should be vessels equipped with bow thruster and some with Controllable Pitch Propeller. F Recommended Geographical Areas:- 1. Dover Straits 2. Singapore Straits 3. Malacca Straits 4. Gibraltar Straits 5. Approaches to New York 6. Approaches to Rotterdam / Flushing 7. Bisan Seto / Kanmon Kaikyo 8. St. Lawrence River 9. Entrance to Mississippi River / approaches to Houston 10. Open Sea Annexure 2 COURSE OUTLINE Training Area; Hours. Lecture; Simulator. 1.; Review of basic principles; 2.0. 2.; Familiarization with the Bridge; 1.0. 3.; Standard Manoeuvres; 0.5; 2.0. 4.; Human Factor in error; 1.0. ■ Attitude, Cultural Awareness, Communication, Briefing & Debriefing and Challenge & Response. ■ Authority – Assertiveness, Management on the bridge ■ Workload and Stress, Decision making. 5.; Passage Planning; 0.5 (exercise). 6.; Wind and current effects; 0.5; 3.5. 7.; Shallow-water effects and Bank, channel and interaction effects; 0.5; 4.0. 8.; Anchoring; 0.5; 3.0. 9.; Single – buoy mooring; 2.0. 10.; Picking up Pilot, Berthing / Unberthing, Use of tugs; 0.5; 8.0. 11.; Crisis Management, Contingency Planning and carrying out a voyage in emergency situations; 0.5; 5.0. TOTAL; 6; 29. GRAND TOTAL; 35. Annexure 3 COURSE TIMETABLE TIME; SUBJECT; FACULTY; TIME; SUBJECT; FACULTY. Group A; Group B. DAY 1. 0900 -1100; Course Entry formalities.; 0900 -1100; Course Entry formalities.. Review of Basic Principles (Lecture); Review of Basic Principles (Lecture). 1100-1115; Tea Break; 1100-1115; Tea Break. 1115-1215; Familiarisation (simulator); 1115-1145; Standard manoeuvre (Lecture). 1215-1245; Standard manoeuvre (Lecture); 1145-1215; Wind & Current (Lecture). 1245-1315; Wind & Current (Lecture); 1215-1315; Familiarisation (simulator). 1315- 1400; Lunch Break; 1315–1400; Lunch Break. 1400-1415; Standard manoeuvre (Plan & Brief); 1400-1500; Human Factor in Errors (Lecture). 1415-1530; Standard manoeuvre Exercise (simulator); 1500- 1515; Standard manoeuvre (Plan & Brief). 1530-1545; Tea Break; 1515-1530; Tea Break. 1545-1615; Standard manoeuvre (Debrief); 1530 -1645; Standard manoeuvre Exercise (simulator). 1615-1645; Ship handling-1 (Video). 1645 -1715; Wind & Current exercise (Plan & Brief); 1645- 1715; Standard manoeuvre (Debrief).. DAY 2. 0900-1045; Wind & Current exercise (simulator); 0900-0930; Ship handling-1 (Video). 0930-1000; Passage planning (Planning Exercise). 1000-1030; Wind & Current exercise (Plan & Brief). 1045-1100; Tea Break; 1030-1045; Tea Break. 1100-1130; Wind & Current exercise (debrief); 1045-1230; Wind & Current exercise (simulator). 1130-1200; Passage planning (Planning Exercise). 1200-1230; Anchoring (Lecture). 1230-1300; Anchoring Exercise (Plan & Brief); 1230-1300; Wind & Current exercise (debrief). 1300-1345; Lunch Break; 1300-1345; Lunch Break. 1345-1515; Anchoring Exercise (simulator); 1345-1415; Ship Handling – 2 (Video). 1415-1445; Anchoring (Lecture). 1445-1515; Anchoring Exercise (Plan & Brief). 1515-1530; Tea Break; 1515-1530; Tea Break. 1530-1600; Anchoring Exercise (Debrief); 1530-1700; Anchoring Exercise (simulator). 1600-1700; Human Factor errors (Lecture). 1700-1730; Single Buoy Mooring Exercise (Plan & Brief); 1700-1730; Anchoring Exercise (Debrief). DAY 3. 0900-1000; Single Buoy Mooring Exercise (simulator); 0900-0930; Shallow water effect (Lecture) Bank channel & Interaction effects (Lecture). 1000-1030; Single Buoy Mooring Exercise (Debrief); 0930-1000; Single Buoy Mooring Exercise (Plan & Brief). 1030-1100; Shallow water effect (Lecture) Bank channel & Interaction effects (Lecture); 1000-1100; Single Buoy Mooring Exercise (simulator). 1100-1115; Tea Break; 1100-1115; Tea Break. 1115-1145; Shallow Water and Interaction Exercise (Plan & Brief); 1115-1145; Single Buoy Mooring Exercise (Debrief). 1145-1315; Shallow and Interaction Exercise (simulator); 1145-1215; Shallow water and Interaction Exercise (Plan & Brief). 1215-1245; Voyage Planning (Video). 1315-1400; Lunch Break; 1245-1330; Lunch Break. 1400-1430; Shallow water and Interaction Exercise (Debrief); 1330-1500; Shallow water and Interaction Exercise (simulator). 1430-1500; Berthing / Unberthing (Picking up Pilot Lecture). 1500-1530; Berthing / Unberthing (Plan & Brief); 1500-1530; Shallow water and Interaction Exercise (Debrief). 1530-1545; Tea Break; 1530-1545; Tea Break. 1545-1730; Berthing / Unberthing Exercise 1 (simulator); 1545-1615; Berthing / Unberthing (Picking up Pilot Lecture). 1615-1645; Berthing / Unberthing (Plan & Brief). 1645-1715; Bridge Team Management (Video ). DAY 4. 0900-0930; Berthing / Unberthing (Debrief); 0900-1045; Berthing / Unberthing Exercise 1 (simulator). 0930-1000; Ship Handling 2 (Video). 1000-1030; Berthing Exercise 2 (Plan + Brief). 1030-1045; Tea Break; 1045-1100; Tea Break. 1045-1230; Berthing Exercise 2 (simulator); 1100-1130; Berthing / Unberthing (Debrief). 1130-1200; Crisis Management (Lecture). 1200-1230; Berthing Exercise 2 (Plan + Brief). 1230-1315; Lunch Break; 1230-1315; Lunch Break. 1315-1345; Berthing Exercise 2 (Debrief); 1315-1500; Berthing Exercise 2 (simulator). 1345-1415; Voyage Planning (Video); 1500-1530; Berthing Exercise 2 (Debrief). 1415-1445; Unberthing Exercise (Plan + brief). 1445-1500; Tea Break; 1530-1545; Tea Break. 1500-1645; Unberthing Exercise (simulator); 1545-1615; Master-Pilot Relationship (Video). 1615-1645; Unberthing Exercise (Plan + brief). 1645-1715; Unberthing Exercise (debrief); 1645-1715; Emergency Manoeuvres (Video). DAY 5. 0900-0930; Crisis Management (lecture); 0900-1045; Unberthing Exercise (simulator). 0930-1000; Master-Pilot Relationship(Video). 1000-1030; Crisis Management Exercise (Plan + Brief). 1030-1045; Tea Break; 1045-1100; Tea Break. 1045-1300; Crisis Management Exercise (simulator); 1100-1130; Unberthing Exercise (Debrief). 1130-1200; Crisis Management Exercise (Plan + Brief). 1200-1230; Case Study. 1300-1345; Lunch Break; 1230-1315; Lunch Break. 1345-1415; Crisis Management Exercise (Debrief); 1315-1530; Crisis Management Exercise (simulator). 1415-1445; Emergency Manoeuvres (Video). 1445-1515; Case Study. 1515-1530; Tea Break; 1530-1545; Tea Break. 1530-1615; Bridge Team Management (Video); 1545-1615; Crisis Management Exercise (Debrief). 1615-1700; Assessment; 1615-1700; Assessment. 1700-1715; Feedback & Certificates; 1700-1715; Feedback & Certificates. Annexure 4 - Course Certificate |
| Back to Training Circulars main page |  |